Assignment – 4

Write code and connections in wokwi for ultrasonic sensor. Whenever distance is less than 100cms send “alert” to ibm cloud and display in device recent events

| Team ID | PNT2022TMID40035 |
| --- | --- |
| Project Name | IOT-based smart crop protection system for agriculture |

# PROGRAM :

// ARDUINO PINS (TRIGGER PIN, ECHO

PIN) const int TRIG_PIN = 7; const int ECHO_PIN = 8;

// Anything over 400 cm (23200 us pulse) is "out of range" const unsigned int max_dist = 23200;

void setup()

{

// The Trigger pin will tell the sensor to range find pinMode(TRIG_PIN, OUTPUT);

digitalWrite(TRIG_PIN, LOW);

//Set Echo pin as input to measure the time duration of pulse returning back from the distance sensor pinMode(ECHO_PIN, INPUT);

// We'll use the serial monitor to view the sensor output Serial.begin(9600);

}

void loop() {

unsigned long t1; unsigned long t2; unsigned long pulse_width; float cm;  float inches;

// Hold the trigger pin high for at least 10 us digitalWrite(TRIG_PIN, 	HIGH);

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);   // Wait for pulse on echo pin  while ( digitalRead(ECHO_PIN) == 0 );   // Measure how long the echo pin was held high (pulse width) // Note: the micros() counter will overflow after ~70 min t1 = micros(); while ( digitalRead(ECHO_PIN) == 1); t2 = micros();  pulse_width = t2 - t1;

// Calculate distance in centimeters and inches. The constants // are found in the datasheet, and calculated from the assumed speed //of sound in air at sea level (~340 m/s).

cm = pulse_width / 58.0;

inches = pulse_width / 148.0;

// Print out results

if ( pulse_width > max_dist ) {

Serial.println("Out of range");

} else {

Serial.println("*******************************");

Serial.print("Distance Measured in cm : ");

Serial.println(cm);   if(cm<100){

// while(true){

Serial.println("Alert !!");

// }

}

Serial.print("*******************************");

}

// Wait at least 1000ms before next measurement delay(1000);

}

# OUTPUT :

Project Link : https://wokwi.com/projects/346290927428436563